Test Plan Timeline RFDF

Test 1: Anechoic Chamber 9/14/26

Primary objectives:

- Verify that the KrakenSDR can pick up 915 MHz signal is anechoic chamber

- Show the signal clearly in the spectrum

- Maintain coherence/calibration across the five receiver channels.

Potential Objectives:

- Compare performance of Master and Slave Kraken

1st Test Plan

| Test | Setup | What to record | Pass criteria |
| --- | --- | --- | --- |
| Baseline | Kraken on, transmitter off | Noise floor / spectrum screenshot | No strong unexpected signal at 915 MHz |
| Master detection | Master Kraken, transmitter on at 915 MHz | Peak frequency, signal level, SNR, screenshot | Clear repeatable peak near 915 MHz |
| Master OFF/ON check | Toggle transmitter OFF/ON | Spectrum change | Peak disappears OFF and returns ON |
| Slave detection | Same setup, swap to Slave Kraken | Peak frequency, signal level, SNR, screenshot | Clear repeatable peak near 915 MHz |
| Slave OFF/ON check | Toggle transmitter OFF/ON | Spectrum change | Same behavior |
| Comparison | Same TX power, distance, orientation, settings | Master vs. Slave signal level/SNR | Both detect signal; differences documented |

Master and Slave Kraken Comparison:

| Parameter | Master | Slave | Difference |
| --- | --- | --- | --- |
| Signal peak | ___ dB | ___ dB | ___ dB |
| Noise floor | ___ dB | ___ dB | ___ dB |
| SNR | ___ dB | ___ dB | ___ dB |
| Detected frequency | ___ MHz | ___ MHz | ___ |
| Detection stable? | Y/N | Y/N |  |